- Develop an 8-bit Microprocessor program to perform 8-bit multiplication and division (A = 14H, B = 06H) and 16-bit multiplication and division (X = 1357H, Y = 0025H) using repeated addition and repeated subtraction. Store the product, quotient and remainder separately and compare the number of loop iterations in each case. Repeat for A = F2H, B = 0AH, X = 9A23H, Y = 0011H.

FULL CODE

;====================================================

; 8085 MULTIPLICATION AND DIVISION

; Repeated Addition and Repeated Subtraction

SET 1:

; A = 14H, B = 06H

; X = 1357H, Y = 0025H

SET 2:

; A = F2H, B = 0AH

; X = 9A23H, Y = 0011H

Multiplication: Repeated Addition

; Division: Repeated Subtraction

;====================================================

;====================================================

; SET 1: 8-BIT MULTIPLICATION

; 14H * 06H = 0078H

; Result: 2050H-2051H (LSB-MSB)

;====================================================

LXI H,0000H

MVI B,14H

MVI C,06H

MUL8A:  MOV A,L

ADD B

MOV L,A

JNC M8AN

INR H

M8AN:   DCR C

JNZ MUL8A

SHLD 2050H

;====================================================

; SET 1: 8-BIT DIVISION

; 14H / 06H = Q=03H, R=02H

;====================================================

MVI A,14H

MVI B,06H

MVI C,00H

DIV8A:  CMP B

JC D8AD

SUB B

INR C

JMP DIV8A

D8AD:   STA 2053H

MOV A,C

STA 2052H

;====================================================

; SET 1: 16-BIT MULTIPLICATION

; 1357H * 0025H = 0002E4B3H

; Result: 2054H-2057H (LSB to MSB)

;====================================================

LXI H,0000H

LXI D,1357H

MVI B,25H

LXI SP,3000H

MUL16A: PUSH B

DAD D

POP B

JNC M16AN

; Carry into upper word required

M16AN:  DCR B

JNZ MUL16A

SHLD 2054H

;====================================================

; SET 1: 16-BIT DIVISION

; 1357H / 0025H = Q=0082H, R=0011H

;====================================================

LXI H,1357H

LXI D,0025H

LXI B,0000H

DIV16A: MOV A,H

CMP D

JC D16AD

JNZ SUB16A

MOV A,L

CMP E

JC D16AD

SUB16A: MOV A,L

SUB E

MOV L,A

MOV A,H

SBB D

MOV H,A

INX B

JMP DIV16A

D16AD:  MOV A,C

STA 2058H

MOV A,B

STA 2059H

SHLD 205AH

;====================================================

; SET 2: 8-BIT MULTIPLICATION

; F2H * 0AH = 0964H

; Result: 2060H-2061H

;====================================================

LXI H,0000H

MVI B,F2H

MVI C,0AH

MUL8B:  MOV A,L

ADD B

MOV L,A

JNC M8BN

INR H

M8BN:   DCR C

JNZ MUL8B

SHLD 2060H

;====================================================

; SET 2: 8-BIT DIVISION

; F2H / 0AH = Q=18H, R=02H

;====================================================

MVI A,F2H

MVI B,0AH

MVI C,00H

DIV8B:  CMP B

JC D8BD

SUB B

INR C

JMP DIV8B

D8BD:   STA 2063H

MOV A,C

STA 2062H

;====================================================

; SET 2: 16-BIT MULTIPLICATION

; 9A23H * 0011H = 000A4263H

; Result: 2064H-2067H

;====================================================

LXI H,0000H

LXI D,9A23H

MVI B,11H

MUL16B: PUSH B

DAD D

POP B

JNC M16BN

; Carry into upper word required

M16BN:  DCR B

JNZ MUL16B SHLD 2064H

;====================================================

; SET 2: 16-BIT DIVISION

; 9A23H / 0011H = Q=0927H, R=000CH

;====================================================

LXI H,9A23H

LXI D,0011H

LXI B,0000H

DIV16B: MOV A,H

CMP D

JC D16BD

JNZ SUB16B

MOV A,L

CMP E

JC D16BD

SUB16B: MOV A,L

SUB E

MOV L,A

MOV A,H

SBB D

MOV H,A

INX B

JMP DIV16B

D16BD:  MOV A,C

STA 2068H

MOV A,B

STA 2069H

SHLD 206AH

HLT